CO3 – AT3 ERROR ANALYSIS TASK

Course Name: DIGITAL SIGNAL PROCESSING

Course Code: ECA0903

Student Name: VANJA VENKATA PRANAY

Register Number: 192412269

Question 40

A sampling rate conversion system converts 16 kHz to 24 kHz using interpolation and decimation factors L = 2 and M = 3. The output sampling frequency obtained is 10.67 kHz. Analyze the implementation, identify the error, and calculate the correct output sampling frequency.

1. Given Data

Input sampling frequency, f_in = 16 kHz

Expected output sampling frequency = 24 kHz

Given interpolation factor, L = 2

Given decimation factor, M = 3

Obtained output sampling frequency = 10.67 kHz

2. Sampling Rate Conversion Formula

For a rational sampling rate conversion system, the output sampling frequency is given by:

f_out = f_in × (L / M)

where L is the interpolation factor and M is the decimation factor.

3. Analysis of the Given Implementation

Using the factors given in the problem, L = 2 and M = 3:

f_out = 16 × (2 / 3) kHz

f_out = 10.67 kHz

Therefore, the obtained value of 10.67 kHz is mathematically correct for L = 2 and M = 3. However, it does not achieve the required conversion from 16 kHz to 24 kHz.

4. Identification of the Error

The implementation error is the incorrect selection of interpolation and decimation factors. To convert 16 kHz to 24 kHz, the required conversion ratio is:

f_out / f_in = 24 / 16 = 3 / 2

Therefore, the correct factors are:

L = 3 and M = 2

The given factors L = 2 and M = 3 are reversed, which reduces the sampling frequency instead of increasing it to 24 kHz.

5. Corrected Solution

Using the corrected interpolation and decimation factors:

f_out = f_in × (L / M)

f_out = 16 × (3 / 2)

f_out = 24 kHz

Therefore, the correct output sampling frequency is 24 kHz.

6. Correct Processing Method

Input signal at 16 kHz

Interpolate by L = 3 → Sampling frequency becomes 48 kHz

Apply a low-pass interpolation/anti-imaging filter

Decimate by M = 2 → Sampling frequency becomes 24 kHz

Obtain the final output at 24 kHz

7. DSP Justification

Interpolation increases the sampling frequency by inserting additional samples and then using a low-pass filter to remove spectral images. Decimation reduces the sampling frequency and requires anti-aliasing filtering before downsampling. In rational sampling rate conversion, the overall sampling-rate change is determined by the ratio L/M. Hence, for conversion from 16 kHz to 24 kHz, the ratio must be 3/2. Using L = 2 and M = 3 gives the ratio 2/3 and therefore produces 10.67 kHz.

8. Correct MATLAB Code

clc;
clear;
close all;
fs_in=16000;
L=3;
M=2;
fs_out=fs_in*L/M;
fprintf('Input Sampling Frequency = %.2f kHz\n',fs_in/1000);
fprintf('Interpolation Factor L = %d\n',L);
fprintf('Decimation Factor M = %d\n',M);
fprintf('Output Sampling Frequency = %.2f kHz\n',fs_out/1000);

9. Expected MATLAB Output

Input Sampling Frequency = 16.00 kHz
Interpolation Factor L = 3
Decimation Factor M = 2
Output Sampling Frequency = 24.00 kHz

10. Conclusion

The sampling rate conversion formula was used to analyze the system. The obtained output of 10.67 kHz is correct only for the given factors L = 2 and M = 3. However, these factors do not perform the required conversion from 16 kHz to 24 kHz. The implementation error is the reversal of the interpolation and decimation factors. By using L = 3 and M = 2, the correct output sampling frequency of 24 kHz is obtained.